Graduation Project Documentation

FortiOS 7.6 Administrator

Building a complete simulated network between two branches using FortiGate devices

| Organization | Creativa |
| --- | --- |
| Project Period | August 11 - October 25, 2026 |
| Discussion Date | September 20, 2026 |
| Environment | EVE-NG Community Edition |
| Device Type | FortiGate-VM64-KVM v7.6.7 |

# 1. Project Overview

The goal of this project is to apply the skills learned in configuring FortiGate devices and confirm understanding of core cybersecurity concepts, by building a complete simulated network connecting two virtual company branches, each equipped with an independent FortiGate device.

## Core Requirements

- At least two FortiGate devices

- A WAN connection between the two branches

- At least two end devices

## Required Tasks

| Status | Task | # |
| --- | --- | --- |
| Completed | Account Configuration | 1 |
| Completed | Interface Configuration | 2 |
| Completed | Firewall Policy | 3 |
| Completed | Routing | 4 |
| Completed | Authentication | 5 |
| Completed | Two Security Profiles | 6 |
| Completed | VPN | 7 |
| Completed (Bonus) | SD-WAN | 8 |

# 2. Network Topology

The network was built inside EVE-NG Community Edition, consisting of two FortiGate devices representing two company branches, connected via a direct WAN link. Each device is also connected to two VPCS devices representing the end-user devices inside each branch.

## Network Diagram

The diagram below shows the complete design after adding a second WAN link (WAN-Link2) to support the bonus SD-WAN task, giving the two branches two independent connection paths.

[ WAN-Link2 (Cloud) ] / \ port4 port4 / \ [VPC3] [VPC4] Fortinet1 --- port1==WAN1==port1 --- Fortinet2 [VPC5] [VPC6] \ / ¦ \ / ¦ \ / eth0 eth0 port2 port3 port2 port3 eth0 eth0 WAN1 (primary) : Fortinet1 10.0.0.1/24 <-> Fortinet2 10.0.0.2/24 WAN2 (SD-WAN) : Fortinet1 20.0.0.1/24 <-> Fortinet2 20.0.0.2/24

## IP Addressing Table

| Device | Interface | IP / Network |
| --- | --- | --- |
| Fortinet1 | port1 (WAN1) | 10.0.0.1/24 |
| Fortinet1 | port2 | 192.168.10.1/24 |
| Fortinet1 | port3 | 192.168.20.1/24 |
| Fortinet1 | port4 (WAN2 - SD-WAN) | 20.0.0.1/24 |
| Fortinet2 | port1 (WAN1) | 10.0.0.2/24 |
| Fortinet2 | port2 | 192.168.30.1/24 |
| Fortinet2 | port3 | 192.168.40.1/24 |
| Fortinet2 | port4 (WAN2 - SD-WAN) | 20.0.0.2/24 |
| VPC3 | eth0 | 192.168.10.10/24 |
| VPC4 | eth0 | 192.168.20.10/24 |
| VPC5 | eth0 | 192.168.30.10/24 |
| VPC6 | eth0 | 192.168.40.10/24 |

## Purpose of Each Task (Quick Overview)

| Task | Security / Functional Purpose |
| --- | --- |
| Account Configuration | Defines who can administer the device and with what privilege |
| Interface Configuration | Assigns an IP address to every port on the device |
| Routing | Tells the device how to reach remote networks |
| Firewall Policy | Allows or blocks traffic between networks |
| Authentication | Verifies user identity before allowing outbound access |
| Security Profiles | Inspects traffic for viruses and unsafe sites |
| VPN | Encrypts the connection between the two branches over the public internet |
| SD-WAN | Keeps connectivity alive even if one WAN link fails |

# 3. Task Implementation Details

## 1. Account Configuration

An additional admin account (nadia-admin) with super_admin privileges was created on both devices, alongside the default admin account.

config system admin edit "nadia-admin" set password <strong-password> set accprofile "super_admin" end

The same steps were repeated on Fortinet2 with the same username.

## 2. Interface Configuration

All used ports were switched from dynamic mode (DHCP) to static mode, and each was assigned a fixed IP as shown in the addressing table above.

config system interface edit "port1" set mode static set ip 10.0.0.1 255.255.255.0 set allowaccess ping https ssh next edit "port2" set mode static set ip 192.168.10.1 255.255.255.0 set allowaccess ping https ssh next edit "port3" set mode static set ip 192.168.20.1 255.255.255.0 set allowaccess ping https ssh end

## 3. Routing

Static routes were added on each FortiGate so it can reach the internal network behind the other device, via the WAN gateway.

config router static edit 1 set dst 192.168.30.0 255.255.255.0 set gateway 10.0.0.2 set device "port1" next edit 2 set dst 192.168.40.0 255.255.255.0 set gateway 10.0.0.2 set device "port1" end # Same steps repeated on Fortinet2 (reversed direction)

## 4. Firewall Policy

Two core policies were created on each device, allowing traffic from the internal network to WAN and from WAN back to the internal network, with NAT enabled.

config firewall policy edit 1 set name "Internal-to-WAN" set srcintf "port2" "port3" set dstintf "port1" set srcaddr "all" set dstaddr "all" set action accept set schedule "always" set service "ALL" set nat enable next edit 2 set name "WAN-to-Internal" set srcintf "port1" set dstintf "port2" "port3" set srcaddr "all" set dstaddr "all" set action accept set schedule "always" set service "ALL" end

## 5. Authentication

A local user (student1) was created and added to a user group (AuthGroup). The group was then attached to the first firewall policy (Internal-to-WAN), so a login is required before traffic from the internal network to WAN is allowed.

config user local edit "student1" set type password set passwd <password> end config user group edit "AuthGroup" set member "student1" end config firewall policy edit 1 set groups "AuthGroup" end

## 6. Security Profiles (Two)

Two security profiles were enabled on the first firewall policy: an Antivirus profile, and an SSL/SSH certificate inspection profile.

config firewall policy edit 1 set utm-status enable set av-profile "default" set webfilter-profile "default" set ssl-ssh-profile "certificate-inspection" end

## 7. VPN (IPsec Site-to-Site)

A Site-to-Site IPsec VPN tunnel was created between the two branches over the WAN link, consisting of two phases (Phase 1 and Phase 2).

### On Fortinet1:

config vpn ipsec phase1-interface edit "VPN-F1-F2" set interface "port1" set peertype any set proposal des-sha256 set remote-gw 10.0.0.2 set psksecret <shared-secret> end config vpn ipsec phase2-interface edit "VPN-F1-F2-P2" set phase1name "VPN-F1-F2" set proposal des-sha256 set src-subnet 192.168.10.0 255.255.255.0 set dst-subnet 192.168.30.0 255.255.255.0 end # Additional route & policy to route traffic through the tunnel: config router static edit 3 set dst 192.168.30.0 255.255.255.0 set device "VPN-F1-F2" end config firewall policy edit 3 set name "VPN-Traffic" set srcintf "port2" "port3" set dstintf "VPN-F1-F2" set srcaddr "all" set dstaddr "all" set action accept set schedule "always" set service "ALL" end

The same steps were repeated on Fortinet2, with the remote-gw and src/dst-subnet addresses reversed.

### Verifying the tunnel status (Phase 1 established):

FortiGate-VM64-KVM # diagnose vpn ike gateway list name: VPN-F1-F2 addr: 10.0.0.1:500 -> 10.0.0.2:500 IKE SA: created 1/1 established 1/1 time 9040/9040/9040 ms status: established

# 4. Bonus Task - SD-WAN

SD-WAN was fully implemented by adding an independent second WAN link (WAN2) alongside the primary link (WAN1), and grouping both inside a single SD-WAN Zone called virtual-wan-link. The idea is that the firewall no longer relies on a single connection only; it distributes the decision across two links, so if one fails, connectivity continues automatically through the other (Failover) — something not achievable with a single WAN link as in the basic setup.

### Step 1: Adding a second WAN link to the network

A new network object named WAN-Link2 was added inside EVE-NG and connected to port4 on both Fortinet1 and Fortinet2, representing a second, independent internet provider separate from the primary link (port1).

| Network | IP | Port | Device |
| --- | --- | --- | --- |
| WAN2 | 20.0.0.1/24 | port4 | Fortinet1 |
| WAN2 | 20.0.0.2/24 | port4 | Fortinet2 |

config system interface edit "port4" set mode static set ip 20.0.0.1 255.255.255.0 set allowaccess ping https ssh end # Fortinet2 uses 20.0.0.2 on the same interface

### Step 2: Creating the SD-WAN Zone and adding both links

SD-WAN was enabled and a zone named virtual-wan-link was created, then port4 (WAN2) was added as a member. The primary link port1 remains the backbone connection between the two branches (also used by the VPN tunnel), while port4 acts as a parallel/backup link within the same zone.

config system sdwan set status enable config zone edit "virtual-wan-link" next end config members edit 1 set interface "port4" set gateway 20.0.0.2 next end end

The same steps were repeated on Fortinet2 (with gateway 20.0.0.1).

### Step 3: Linking the firewall policy to the SD-WAN zone

The first policy (Internal-to-WAN) was modified so its outgoing interface (dstintf) is the entire SD-WAN zone (virtual-wan-link) instead of a single fixed port (port1). This way, the firewall automatically selects the appropriate path among the available links inside the zone.

config firewall policy edit 1 set dstintf "virtual-wan-link" end

### SD-WAN Members Table After Configuration

| # | Port | Gateway | Status |
| --- | --- | --- | --- |
| 1 | port4 | 20.0.0.2 (on Fortinet1) | Up |

The member status was verified with the diagnose sys sdwan member command, confirming that port4 is correctly registered inside SD-WAN with the correct gateway.

FortiGate-VM64-KVM # diagnose sys sdwan member Member(1) : interface: port4, gateway: 20.0.0.2, priority: 1, weight: 0

# 5. Testing and Proof the Network Works

Testing was performed in two stages: first after completing the seven core tasks (including VPN), and second after adding SD-WAN and updating the firewall policy to use the SD-WAN zone, to confirm the connection was not affected by the change.

### First Test: After VPN Setup

VPCS> ping 192.168.30.10 192.168.30.10 icmp_seq=1 ttl=62 time=1.2 ms (reply) 192.168.30.10 icmp_seq=2 ttl=62 time=0.9 ms (reply) 192.168.30.10 icmp_seq=3 ttl=62 time=1.0 ms (reply) # Result: Successful connection between branches via VPN tunnel

### Second Test: After Enabling SD-WAN and Linking the Policy

After changing dstintf in the first policy from port1 to virtual-wan-link (the SD-WAN zone), the ping test was repeated between all end-user devices in both branches (VPC3, VPC4 with VPC5, VPC6) to confirm the connection remained stable.

| Source | Destination | Result |
| --- | --- | --- |
| VPC3 | VPC5 (192.168.30.10) | Success |
| VPC4 | VPC6 (192.168.40.10) | Success |
| All devices | All other devices | Success |

The result confirms that traffic between the two branches actually passes through the IPsec VPN tunnel via the SD-WAN zone, and that all configurations (routing, firewall policies, authentication, SD-WAN) work together seamlessly — not just theoretical settings.

### Operational Note: Saving End-Device (VPCS) Configuration

It was observed that VPCS devices (unlike FortiGate devices) lose their manually entered IP address after any restart. To keep the network ready for the discussion without needing to re-enter it, the save command was used after every IP assignment on each VPCS device, so the same address is automatically restored whenever the device starts again.

VPCS> ip 192.168.10.10/24 192.168.10.1 VPCS> save # Repeat both commands on every VPCS with its own address

# 6. Conclusion

A complete simulated network was built between two branches using two FortiGate devices, and all required project tasks were successfully implemented: account setup, interfaces, routing, firewall policies, authentication, security profiles, and a VPN tunnel between the branches, in addition to the bonus SD-WAN task with two independent links (WAN1 and WAN2). The network was practically verified through successful ping tests between the branches, both before and after adding SD-WAN, while ensuring end-device configurations remain persistent across restarts.